SENTRY

Machine Learning

| Item | Current project specification |
| --- | --- |
| Sensors | MQ-2, MQ-3, MQ-135, SEN0567 (NH3) |
| Sampling | 100 ms; 30 samples; 3-second window |
| Features | 7: VMQ2, VMQ3, VMQ135, VSEN0567, dV/dt_max, temperature, humidity |
| Classes | SAFE, WEATHER, ALCOHOL, EXPLOSIVE, NARCOTIC |
| Deployment model | 7 → Dense(16, ReLU) → Dense(8, ReLU) → Dense(5, softmax), INT8 TFLite |

# 1. What happens to the sensor input?

SENTRY does not make a decision from one sensor reading. It collects a short time window from four gas sensors plus temperature and humidity. The readings are converted into a consistent model space, environmentally compensated, reduced into seven features, standardized with the saved training scaler, and passed to the classifier.

Acquire sensor readings.

Collect 30 samples at 100 ms intervals.

Convert MQ raw values to model-space values.

Apply temperature/humidity compensation.

Calculate the early dV/dt_max feature.

Average the last five samples for final gas/T/H values.

Build the seven-feature vector in the fixed order.

Apply the saved StandardScaler.

Run the ML model and decode one of five classes.

# 2. Raw input and sampling

Each inference window contains 30 samples separated by 0.1 s, giving a 3-second observation window. Each sample contains four gas channels plus temperature and humidity.

Conceptually: [MQ2(t), MQ3(t), MQ135(t), SEN0567(t), T(t), H(t)], t = 1…30.

# 3. MQ baseline conversion

The ESP32 raw values are not used directly as the model's MQ feature values. For each MQ sensor, the current pipeline maps the raw reading using a real-device baseline and a model-space baseline.

| Sensor | Real baseline | Model baseline |
| --- | --- | --- |
| MQ-2 | 3069.0 | 0.82 |
| MQ-3 | 530.33 | 0.74 |
| MQ-135 | 1043.27 | 0.91 |
| SEN0567 | — | Assumed |

V_model = (Raw / RealBaseline) × ModelBaseline

Therefore, for example, MQ-2 uses (Raw / 3069.0) × 0.82. SEN0567 currently assumes model-space voltage; if the hardware sends ADC counts, a corresponding conversion must be added.

# 4. Environmental compensation

Temperature and humidity can shift sensor response. The prototype compensates each gas channel relative to 25 °C and 50% RH.

V_comp = V_raw − α_H(H − 50) − α_T(T − 25)

| Sensor | α_H | α_T |
| --- | --- | --- |
| MQ-2 | 0.0018 | 0.0008 |
| MQ-3 | 0.0014 | 0.0007 |
| MQ-135 | 0.0022 | 0.0009 |
| SEN0567 | 0.0016 | 0.0008 |

These α values are the current synthetic/prototype compensation parameters. They should be re-estimated from controlled measured data before safety-critical use.

# 5. Seven-feature extraction

X = [VMQ2, VMQ3, VMQ135, VSEN0567, dV/dt_max, T, H]

| Feature | Construction | Purpose |
| --- | --- | --- |
| VMQ2 | Mean of last 5 processed samples | Gas level pattern |
| VMQ3 | Mean of last 5 processed samples | Gas/alcohol-sensitive pattern |
| VMQ135 | Mean of last 5 processed samples | Broad gas/air-quality pattern |
| VSEN0567 | Mean of last 5 processed samples | NH3-sensitive pattern |
| dV/dt_max | Max absolute early slope across gas channels | Captures rapid changes |
| Temperature | Mean of last 5 samples | Environmental context |
| Humidity | Mean of last 5 samples | Environmental context |

# 6. dV/dt: capturing how fast a gas event starts

The first 300 ms are used for the dynamic feature. With 100 ms sampling, the first four samples create three slope intervals.

slope = (V[i+1] − V[i]) / 0.1

dV/dt_max = max(|slope|) across the four gas sensors and early intervals

The absolute value means both fast rises and fast falls can contribute.

# 7. Last-five-sample averaging

The final gas, temperature and humidity values use the mean of the last five readings, corresponding to the final 500 ms. This reduces sensitivity to a single noisy sample while preserving the end-of-window state.

V_final = mean(last 5 samples)

# 8. Standardization

The seven features have different numerical ranges. StandardScaler is fitted only on the training split, and its learned parameters are saved and reused during inference.

z = (x − μ) / σ

μ and σ are the training-set mean and standard deviation for each feature. Keeping these values fixed is essential for training/inference parity.

# 9. Dataset and training setup

| Dataset | Size | Class balance | Use |
| --- | --- | --- | --- |
| Synthetic training dataset | 60,000 | 12,000/class | Training + validation |
| Blind final test | 12,000 | 2,400/class | Final evaluation only |

The checked audit reports no NaN/infinite values or duplicates, no train/test overlap, and an 80/20 stratified training/validation split with random_state = 42. The final test set is not used for training.

# 10. Models benchmarked

| Model | Role |
| --- | --- |
| Random Forest | Classical ensemble baseline |
| XGBoost | Gradient-boosted tree benchmark |
| HistGradientBoosting | Histogram gradient boosting benchmark |
| SVM RBF | Non-linear kernel baseline |
| Logistic Regression | Linear baseline |
| KNN | Distance-based baseline |
| Extra Trees | Randomized tree ensemble |
| TinyML MLP | Compact neural model for embedded inference |

# 11. Random Forest configuration

RandomForestClassifier(n_estimators=150, max_depth=8, random_state=42, n_jobs=-1)

# 12. TinyML architecture and quantization

| Layer | Configuration |
| --- | --- |
| Input | 7 features |
| Dense 1 | 16 neurons, ReLU |
| Dense 2 | 8 neurons, ReLU |
| Output | 5 neurons, softmax |

The neural network is converted to TensorFlow Lite and quantized to INT8. The current INT8 artifact is 3,608 bytes (about 3.52 KB), with INT8 input and output. Representative calibration data uses 500 training samples.

Tree models such as XGBoost are not directly converted into TFLite neural-network models. If an XGBoost-style model is targeted for an MCU, it needs a separate compact-tree/C deployment approach.

# 13. Safety-oriented evaluation

For safety analysis, EXPLOSIVE + NARCOTIC are treated as THREAT, while SAFE + WEATHER + ALCOHOL are NON-THREAT.

| Metric term | Meaning |
| --- | --- |
| True Positives | Actual threat predicted as threat |
| False Negatives | Actual threat predicted as non-threat |
| False Positives | Actual non-threat predicted as threat |
| True Negatives | Actual non-threat predicted as non-threat |

Threat Recall = TP / (TP + FN)

Threat FNR = FN / (TP + FN)

The project tracks FN/FNR separately because a missed threat is more important to identify than accuracy alone in this safety framing. This is a project requirement, not a claim of certified safety performance.

# 14. Blind-test benchmark results

These are the reported results from the uploaded benchmark on the 12,000-sample blind final test.

| Model | Accuracy | Macro F1 | Threat Recall | Threat FNR | FN |
| --- | --- | --- | --- | --- | --- |
| XGBoost | 90.45% | 90.48% | 93.21% | 6.79% | 326 |
| HistGradientBoosting | 90.37% | 90.40% | 93.10% | 6.90% | 331 |
| KNN | 87.95% | 88.03% | 92.17% | 7.83% | 376 |
| SVM RBF | 89.59% | 89.64% | 91.90% | 8.10% | 389 |
| Random Forest | 89.46% | 89.51% | 89.88% | 10.13% | 486 |
| Logistic Regression | 84.29% | 84.37% | 87.42% | 12.58% | 604 |
| Extra Trees | 83.91% | 84.10% | 72.77% | 27.23% | 1307 |

These values describe performance on this particular synthetic blind test; they should not be generalized to real-world gas detection without physical validation.

# 15. Per-threat recall in the benchmark

| Model | EXPLOSIVE recall | NARCOTIC recall |
| --- | --- | --- |
| XGBoost | 92.96% | 87.75% |
| HistGradientBoosting | 93.33% | 87.63% |
| KNN | 92.38% | 85.42% |
| SVM RBF | 92.58% | 86.25% |
| Random Forest | 90.42% | 84.21% |
| Logistic Regression | 87.42% | 83.38% |
| Extra Trees | 82.79% | 61.33% |

# 16. TinyML results and audit status

| Variant | Reported accuracy | Threat breakdown available in regenerated audit |
| --- | --- | --- |
| TinyML Float32 | 88.28% | TP 4408, TN 6736, FP 464, FN 392; recall 91.83%; FNR 8.17% |
| TinyML INT8 | 88.01% | Detailed threat breakdown not included in the regenerated audit excerpt |

The regenerated audit is marked PASS and reports checks covering dataset integrity, separation, feature calculation, compensation, scaler use, model configuration, final-test isolation, quantization and training/inference parity. It also contains stale duplicated summary text with older TinyML accuracy values (87.90% / 87.70%); those stale lines should be removed before final submission.

# 17. Environmental robustness

The project checks dry, normal, humid, very humid, cold, hot, hot+humid and hot+dry conditions. The checked robustness work shows extreme humidity can make SAFE conditions harder to separate. Temperature and humidity are therefore retained as explicit features, alongside compensation.

# 18. Why these seven features?

The four gas channels give a multi-sensor chemical pattern. The final levels describe the state at the end of the window. dV/dt_max adds early temporal behavior, which can distinguish a rapid change from a slowly changing background. Temperature and humidity provide environmental context. Together these signals give the classifier both spatial/multi-sensor and temporal information.

# 19. Important assumptions

MQ-2, MQ-3 and MQ-135 use real baselines 3069.0, 530.33 and 1043.27.

Model-space baselines are 0.82, 0.74 and 0.91.

SEN0567 is currently assumed to already be model-space voltage.

dV/dt uses 0.1 s intervals and the first four samples.

Final gas/T/H values use the last five samples.

The seven-feature order must remain identical during deployment.

Current α values are prototype coefficients, not lab calibration constants.

Current datasets are synthetic and behavior-inspired.